# First Position Results

All results correspond to t = 0 at the specified initial geometry. The input angle is θAB = 61.8687° from +x; AB rotates counterclockwise at 23.148148 rpm with zero input angular acceleration. The cycle period is 2.592 s.

The global +x direction is rightward and +y is upward. Positive torque acts about +z (counterclockwise). Vertical bars denote vector magnitudes. PMKS+ values are reported to three decimal places.

TABLE I

STATIC EQUILIBRIUM JOINT FORCES AND INPUT TORQUE

| Quantity | Unit | MATLAB | PMKS+ |
| --- | --- | --- | --- |
| FAx | N | 1053.234 | 1053.065 |
| FAy | N | 140.773 | 140.768 |
| ¦FA¦ | N | 1062.600 | 1062.432 |
| FBx | N | −1053.234 | −1053.065 |
| FBy | N | −61.999 | −62.020 |
| ¦FB¦ | N | 1055.058 | 1054.890 |
| FCx | N | −1053.234 | −1053.065 |
| FCy | N | 128.413 | 128.327 |
| ¦FC¦ | N | 1061.034 | 1060.856 |
| FDx | N | −647.911 | −647.774 |
| FDy | N | 526.079 | 525.881 |
| ¦FD¦ | N | 834.595 | 834.363 |
| FEx | N | −405.323 | −405.291 |
| FEy | N | −69.227 | −69.227 |
| ¦FE¦ | N | 411.192 | 411.161 |
| FFx | N | −405.323 | −405.291 |
| FFy | N | 89.302 | 89.248 |
| ¦FF¦ | N | 415.044 | 415.002 |
| FGx | N | −405.323 | −405.291 |
| FGy | N | 874.763 | 874.509 |
| ¦FG¦ | N | 964.104 | 963.861 |
| Tz | N·m | −504.373 | −504.264 |

Note. The entries are joint-force components and magnitudes, plus the required input torque. Link self-weights and a downward 200 N force at H are included. The negative torque denotes a clockwise holding torque. PMKS+ values are native CSV first rows at t = 0 from Static (Equilibrium) force and input-torque exports. FA, FD, and FG are ground reactions. FB, FC, FE, and FF act on AB, BC, DCE, and EF, respectively; each adjacent link experiences the opposite force. PMKS+ uses g = 9.80665 m/s² and MATLAB uses g = 9.81 m/s²; saved coordinate precision also differs between the models.

TABLE II

NEWTON’S SECOND LAW JOINT FORCES AND INPUT TORQUE

| Quantity | Unit | MATLAB |
| --- | --- | --- |
| FAx | N | 143.387 |
| FAy | N | 132.347 |
| ¦FA¦ | N | 195.130 |
| FBx | N | −149.839 |
| FBy | N | −65.640 |
| ¦FB¦ | N | 163.586 |
| FCx | N | −176.936 |
| FCy | N | 80.182 |
| ¦FC¦ | N | 194.257 |
| FDx | N | −130.445 |
| FDy | N | 377.591 |
| ¦FD¦ | N | 399.489 |
| FEx | N | −97.511 |
| FEy | N | −38.534 |
| ¦FE¦ | N | 104.849 |
| FFx | N | −135.745 |
| FFy | N | 39.390 |
| ¦FF¦ | N | 141.345 |
| FGx | N | −222.269 |
| FGy | N | 526.306 |
| ¦FG¦ | N | 571.315 |
| Tz | N·m | −47.175 |

Note. These values use the same constant downward 200 N force at H as the static analysis. Link self-weights and the translational and rotational inertia of all five links are included. No additional artifact mass is inferred from the prescribed force. The Table I force convention applies. No PMKS+ comparison is made.

TABLE III

LINK ANGULAR VELOCITIES AND ACCELERATIONS

| Link | MATLAB ωz (rad/s) | PMKS+ ωz (rad/s) | MATLAB αz (rad/s²) | PMKS+ αz (rad/s²) |
| --- | --- | --- | --- | --- |
| AB | 2.4241 | 2.424 | 0.0000 | 0.000 |
| BC | 0.4895 | 0.490 | −0.9577 | −0.958 |
| DCE | 1.2716 | 1.272 | 1.2785 | 1.278 |
| EF | 1.0834 | 1.083 | 1.4305 | 1.430 |
| GFH | 1.3470 | 1.347 | 1.5039 | 1.504 |

Note. The entries are the signed z components of angular velocity and angular acceleration for each rigid link; the in-plane angular components are zero. Positive values act about +z. The constant input speed gives αAB = 0. PMKS+ references include all five links at t = 0 s.

TABLE IV

JOINT AND POINT LINEAR VELOCITIES

| Quantity | Unit | MATLAB | PMKS+ |
| --- | --- | --- | --- |
| vAx | m/s | 0.0000 | 0.000 |
| vAy | m/s | 0.0000 | 0.000 |
| ¦vA¦ | m/s | 0.0000 | 0.000 |
| vBx | m/s | −1.2242 | −1.224 |
| vBy | m/s | 0.6545 | 0.654 |
| ¦vB¦ | m/s | 1.3881 | 1.388 |
| vCx | m/s | −1.2462 | −1.246 |
| vCy | m/s | −0.0381 | −0.038 |
| ¦vC¦ | m/s | 1.2468 | 1.247 |
| vDx | m/s | 0.0000 | 0.000 |
| vDy | m/s | 0.0000 | 0.000 |
| ¦vD¦ | m/s | 0.0000 | 0.000 |
| vEx | m/s | −3.1600 | −3.160 |
| vEy | m/s | −0.1144 | −0.114 |
| ¦vE¦ | m/s | 3.1620 | 3.162 |
| vFx | m/s | −3.1925 | −3.192 |
| vFy | m/s | −1.3874 | −1.387 |
| ¦vF¦ | m/s | 3.4809 | 3.481 |
| vGx | m/s | 0.0000 | 0.000 |
| vGy | m/s | 0.0000 | 0.000 |
| ¦vG¦ | m/s | 0.0000 | 0.000 |
| vHx | m/s | −5.4693 | −5.469 |
| vHy | m/s | −2.3770 | −2.378 |
| ¦vH¦ | m/s | 5.9635 | 5.963 |

Note. The entries are global x and y components and magnitudes of linear velocity. Grounded joints A, D, and G have zero velocity. H is the load-application point on the rigid GFH extension, not an additional revolute joint. PMKS+ references include all listed points at t = 0 s.

TABLE V

JOINT AND POINT LINEAR ACCELERATIONS

| Quantity | Unit | MATLAB | PMKS+ |
| --- | --- | --- | --- |
| aAx | m/s² | 0.0000 | 0.000 |
| aAy | m/s² | 0.0000 | 0.000 |
| ¦aA¦ | m/s² | 0.0000 | 0.000 |
| aBx | m/s² | −1.5865 | −1.587 |
| aBy | m/s² | −2.9674 | −2.967 |
| ¦aB¦ | m/s² | 3.3649 | 3.365 |
| aCx | m/s² | −1.2044 | −1.204 |
| aCy | m/s² | −1.6230 | −1.623 |
| ¦aC¦ | m/s² | 2.0211 | 2.021 |
| aDx | m/s² | 0.0000 | 0.000 |
| aDy | m/s² | 0.0000 | 0.000 |
| ¦aD¦ | m/s² | 0.0000 | 0.000 |
| aEx | m/s² | −3.0315 | −3.031 |
| aEy | m/s² | −4.1333 | −4.133 |
| ¦aE¦ | m/s² | 5.1258 | 5.126 |
| aFx | m/s² | −1.6952 | −1.695 |
| aFy | m/s² | −5.8493 | −5.849 |
| ¦aF¦ | m/s² | 6.0900 | 6.090 |
| aGx | m/s² | 0.0000 | 0.000 |
| aGy | m/s² | 0.0000 | 0.000 |
| ¦aG¦ | m/s² | 0.0000 | 0.000 |
| aHx | m/s² | −2.9043 | −2.903 |
| aHy | m/s² | −10.0210 | −10.021 |
| ¦aH¦ | m/s² | 10.4334 | 10.433 |

Note. The entries are global linear-acceleration components and magnitudes, including tangential and centripetal contributions. A, D, and G are fixed and have zero acceleration. PMKS+ references include all listed points at t = 0 s.

TABLE VI

ACCELERATIONS AT MASS CENTERS

| Quantity | Unit | MATLAB | PMKS+ |
| --- | --- | --- | --- |
| aS1x | m/s² | −0.8035 | −0.805 |
| aS1y | m/s² | −1.5028 | −1.504 |
| ¦aS1¦ | m/s² | 1.7041 | 1.706 |
| aS2x | m/s² | −1.3961 | −1.397 |
| aS2y | m/s² | −2.2973 | −2.298 |
| ¦aS2¦ | m/s² | 2.6882 | 2.689 |
| aS3x | m/s² | −1.5239 | −1.524 |
| aS3y | m/s² | −2.0778 | −2.077 |
| ¦aS3¦ | m/s² | 2.5767 | 2.576 |
| aS4x | m/s² | −2.3660 | −2.366 |
| aS4y | m/s² | −4.9880 | −4.988 |
| ¦aS4¦ | m/s² | 5.5206 | 5.521 |
| aS5x | m/s² | −1.4498 | −1.450 |
| aS5y | m/s² | −5.0024 | −5.003 |
| ¦aS5¦ | m/s² | 5.2083 | 5.209 |

Note. The entries are global x and y components and magnitudes of mass-center acceleration. S1–S5 belong to AB, BC, DCE, EF, and GFH, respectively. Their locations use the supplied CAD centers mapped into the linkage plane. PMKS+ entries are direct built-in COM acceleration exports at t = 0 s, using the saved CAD centers and the same input speed.